# 3D Printing Summary

| Metrics |  |
| --- | --- |
| Total Print Time (min) | 308 |
| Total Number of Components | 4 |
| Typical Total Mass (g) | 33 |
| Typical Number of Print Setups | 4 |

# 3D Printing Settings

| Print File Name | Qty | Total Print Time (hr:min) | Mass (g) | Infill (%) | Support(Y/N) | Layer Height/ Nozzle Diameter(mm) | Notes (orientation, special settings, etc.) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Grip Handle.stl | 1 | 3:44 | 26 | 20 | N | 0.2/0.4 | - Print in orientation given in STL |
| Grip Insert.stl | 1 | 0:15 | 2 | 20 | N | 0.2/0.4 | - Print in orientation given in STL |
| insert.stl | 1 | 0:27 | 2 | 20 | Y | 0.2/0.4 | - Requires support - Print in orientation given in STL |
| spoon attachment.stl | 1 | 0:32 | 3 | 20 | Y | 0.2/0.4 | - Requires support - Print in orientation given in STL |

# Customization Options

- The device can be printed in any colour the user would like.

# Post-Processing

- Remove support material

- Remove any sharp edges using a hobby knife or sandpaper

# Examples of Quality Prints

| Photo of Grip Handle.stl | Photo of Grip Insert.stl |
| --- | --- |
| Photo of insert.stl | Photo of spoon attachment.stl |