# Required Components

|  | BOM - Grip Handle - Grip insert - Insert - Spoon attachment - Bearing - Fishing weights - Utensil - Computer case screw |
| --- | --- |

# Required Tools

- Tin Snips

- Superglue

- Hot Glue

# Required Personal Protective Equipment (PPE)

- Eye protection

# Assembly Instructions

## Step 1

Use superglue to glue the bearing into the grip handle. Be careful to not get any into the bearing.

## Step 2

Use superglue to attach the insert into the bearing. Be careful not to get any into the bearing.

## Step 3

Cut the utensil at the desired length and hot glue it into the slot on the spoon attachment.

## Step 4

Using the set screw, attach the spoon section to the handle section.

## Step 5

If the user wants a weighted utensil, add the desired amount of weights to the hollow handle section. A tissue can optionally be used to prevent the weights from sliding around in the handle.

## Step 6

Use superglue to attach the grip insert to the end of the handle. Optionally, a soldering iron can be used to melt the two pieces together.